Disjointing Feature Request and Explanation

# The Problem Description

The WQS Reliability Block Diagram and Operation simulation module contains the feature called “Directional or Flow lock Diagram” with possible choice of the Monte Carlo Simulation.

The Flow diagram allows set “direction of flow from input to output and as it has been conceived is built on the same basis as any other block diagram – Boolean algebra, or Boolean Methods.

Boolean methods apply the Boolean logic of success or fail criteria, depending on the preference, With Boolean Logic, a preferred path is failed path,

Boolean method is very good for solving complex and large RBDs or networks like shown on Figure 1 which depicts power distribution to 64 houses in 16 streets, through 4 step down transformers.

In building of blocks there does not seem to be accounting of the common blocks, like the same block common to multiple paths. The success paths seem to treat each repeated block as a separate block of the same kind which flows through the different paths.

There seems to be no option for the calculations such as any Boolean reduction or application of rules, not even to disregard repeated blocks accounted for multiple times, but in fact they are common blocks that cannot fail or survive multiple times.

Option to apply disjointing necessary for the correct results or other methods which adjust simple success or failure criteria additions (rare approximations) to become correct results.

# Boolean Paths (example)

This example is very much like the very simplified actual application of the flow RBDs (Boolean paths.

Figure 1 represents the power distribution that is a very common calculation done by professional engineers to ensure reliability of the electric power grids. A simplified case can be often found on the Professional Electrical Engineer examinations questions.

Figure 1 - Power distribution to 64 houses

Assume that the Utility Company is not allowed to have the power interrupted to more than 6 houses out of 64

This the distribution in Figure 1 would have resulted in a rather complex RBD with:

One independent block, A, common to:

- 4 transformers, B,

- 16 street low voltage transformers C, and

- 64 users – houses.

The second tier, Blocks B are common blocks to:

4 street low voltage transformers C

- 16 houses D

Each of the street transformer C is common (supplies) to

- 4 houses, D.

The scenario then contains 64 paths where Block A repeats 64 times, Block B 16 times and each block C repeats 4 times.

This relatively simple scenario becomes really complex if simulation is involved because then there are cross assumptions which blocks are down and which are up. Thus we will address the straight Boolean approach.

# Boolean Methods and Quantitative Analysis

Boolean approach for probabilistic results, requires quantitative analysis, and answer to a reliability question as straight Boolean approach needs to be completed again assuming failure or success paths.

If Boolean approach of OR situation would apply the success paths would repeat with the speculation or what block is up and which is down. And summing up success states – rare approximation or cut sets summation.

This is the approach that this PTC software applies. However, to get this network reliability results or any correct results, the Boolean rules must be applied:

- “No same block with the same state can be repeated in any two or more paths”, the common blocks

- This must be distinguished from the repeated blocks

There are several ways to do the Boolean reductions:

- Truth table reduction,

  - This is done in accordance with the basic rules of Boolean Algebra:

The example of a system where the success Boolean expression would have been:

- Total probability theorem which would reduce the block diagram to the parallel configurations of parallel blocks made of the cut sets or series configurations or tie sets,

The Sylvester-Poincare formula for total probability which would produce reliability of this system would be:

The truth table for the above system of three parallel blocks would be in Table 1 and the same table as consensus represents the Boolean reduction that leads to the system success state:

, which is then translated to reliability terms making the Reliability as determined by Sylvester Poincare total reliability formula.

Table 2: – Boolean truth table reliability calculation of three parallel blocks

| State number | Item |  |  | System | Disjointed terms | Consensus |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 |  |  |  |
| 2 | 0 | 0 | 1 | 1 |  |  |  |
| 3 | 0 | 1 | 0 | 1 |  |  |  |
| 4 | 0 | 1 | 1 | 1 |  |  |  |
| 5 | 1 | 0 | 0 | 1 |  |  | a |
| 6 | 1 | 0 | 1 | 1 |  |  |  |
| 7 | 1 | 1 | 0 | 1 |  |  |  |
| 8 | 1 | 1 | 1 | 1 |  |  |  |
| NOTE 1= working, 0 = failed. |  |  |  |  |  |  |  |

- Disjointing and other manipulations of the success or failure paths as shown in the attached Pre-release document, the IEC/ANSI 61078, Reliability Block Diagrams and Boolean Methods (cannot provide the released versions to not violate copyright laws) Section 11  and Annex B.

A simple example of disjointing process that we are sure PTC experts are familiar with, can be shown on a simple bridge circuit as shown in the IEC/ANSI 61025, Fault tree analysis, Edition 2 of 2006 attached as well in its pre-release (FDIS) format. The example is shown with for the three different settings of a good FTA software (Rare Approximation, Esary Proschan and Disjointed – correct results. Also, the total probability theorem used for the RBDs is also shown.

The example circuit from this standard is shown in Figure 2.

Figure 2 – Bridge Circuit Example

The total reliability theorem can be worded for this circuit as follows:

The system, if the block E is good, is made of two parallel systems connected in series, Parallel system A, B) and parallel system (C,D) OR

If the Block E failed, the whole system is represented by one parallel system: Branch 1: blocks A and C, and the branch 2, made of blocks B and D. The system reliability equation is:

If the probability of failure values were:

With the Boolean algebra, the system can be addressed in three ways:

Rare approximation, failed paths – drastically incorrect results for this example:

With the Esary-Proschan:

The correct, disjointed by the software or with the total reliability equation:

In the IEC 61025, the disjointing for the example in Figure 2 is shown in Annex B:

B.1 Success approach for disjointing (reference IES/ANSI 6102, FTA)

The following text has been copied directly from the referenced standard.

Suppose the reliability block diagram represents a network consisting of five elements A, B, C, D and E and that a.,b,c,d and e denote the corresponding Boolean "success" variables. It is also supposed that system success (SS) in Boolean terms is defined by the following expression, which comprises four sum-of-product terms:

SS = ac + bd + aed +bec

To make the above expression disjoint, the procedure is as follows:

Step 1.1: Make each term disjoint with respect to the first. Proceed in a systematic way to make the second term disjoint with respect to the first. Inspect the two terms to see if any variable in the first appears in complemented form in the second. If this is so, the two terms are already disjoint and there is nothing further to do. If not, pick out all the variables in the first term (ac), which do not appear in the second (bd). (In set theory terminology this is called the relative complement of the second term with respect to the first). In this particular step the result is the variables a and c.

Step 1.2: Replace the second term bd by abd + acbd

Step 1.3: Make the third term disjoint with respect to the first. First of all, inspect the two terms to see if any variable in the first appears in complemented form in the second. Since this is not so, identify the relative complement of the third term with respect to the first: namely the variable c. Hence replace the third term by the term Caed

Step 1.4: Make the fourth term (bec) disjoint with respect to the first. The relative complement of the fourth term with respect to the first is the variable a. Therefore replace the fourth term by abet . The system success expression at this stage thus becomes:

SS1 = ac+-abd + dad +-c-aed +abec

Now repeat the process starting with the second term.

Step 2.1: Make the third term of SS1 (cad) disjoint with respect to the second (abd). In this instance the terms are already disjoint (on account of variable a), so there is nothing further to do.

Step 2.2: Make the fourth term of SS1 (T;aed) disjoint with respect to the second (abd). In this instance also, note the terms are already disjoint (on account of variable a) so there is nothing further to do.

Step 2.3: Make the fifth term of SS1 (a—bec) disjoint with respect to the second (abd). The relative complement is the variable d so replace the fifth term by dabec

The system success expression at this stage thus becomes:

SS2 =ac+Fibd+cebd+Faed+Gbec

Now repeat the process starting with the third term.

Step 3.1: Make the fourth term of SS2 (coed) disjoint with respect to the third (a-crbd). The relative complement is the variable h so replace the fourth term by hcaed.

Step 3.2: Make the fifth term of SS2 (dabec)disjoint with respect to the third (acbd). In this instance the terms are already disjoint (on account of variables a, c and d), so there is nothing further to do.

The system success expression at this stage is

SS3 = ac+ abd + acbd +bcued +dabec

and since no further simplification appears possible, this is the final disjointed expression.

Making the usual substitutions, the equation for system reliability is given by:

Rsi = Rak + (1— Ra)RbRd + Ra(1— Rc)RbRd + (1— Rb)(1— R,)RaReRci + (1— Rd)(1— Ra)RbReRc

It should be noted that the form of the final result, in this case SS3, will depend on the order in which the terms in the original Boolean expression are written down.

By using the theorem of total probability, it can be shown that the expression for system reliability can be written in the form:

RS2 =	+ Rb — RaRb)(Re Rd — ReRd)Re + (Rai?, + RhRd — RaReRbRdX1 — Re)

Although the expressions for R51 and Rs2 look quite different, they are in fact identical.

Part B.2 of the standard shows disjointing with the failure (cut sets) approach

B.2 System failure, SF, (cut sets) approach

Disjointing is explained as follows:

SF = ab+cd+aed +bec

To make the above expression disjoint, the procedure is as follows:

Step 1.1: Make each term disjoint with respect to the first. Proceed in a systematic way to make the second term disjoint with respect to the first. Inspect the two terms to see if any variable in the first appears in complemented form in the second. If this is so, the two terms are already disjoint and there is nothing further to do. If not, pick out all the variables in the first term (ab), which do not appear in the second (cd). (In set theory terminology this is called the relative complement of the second term with respect to the first.) In this particular step the result is the variables a and b.

Step 1.2: Replace the second term cd by acd+abcd.

Step 1.3: Make the third term disjoint with respect to the first. First of all, inspect the two terms to see if any variable in the first appears in complemented form in the second. Since this is not so, identify the relative complement of the third term with respect to the first:  namely the variable b. Hence replace the third term by the term baed

Step 1.4: Make the fourth term (bac) disjoint with respect to the first. The relative complement of the fourth term with respect to the first is the variable a. Therefore replace the fourth term by dbec. The system failure expression at this stage thus becomes:

SFq = ab + acd + ab cd + baed + bec

Now repeat the process starting with the second term. Hence:

Step 2.1: Make the third term of SF1 (abcd) disjoint with respect to the second (dcd). in this instance, the terms are already disjoint (on account of variable a), so there is nothing further to do.

Step 2.2: Make the fourth term of SF1 (baed) disjoint with respect to the second (dcd). In this instance also, note the terms are already disjoint (on account of variable a) so there is nothing further to do.

Step 2.3: Make the fifth term of SF1 (abec) disjoint with respect to the second (dcd). The relative complement is the variable d so replace the fifth term by aibec.

The system failure expression at this stage thus becomes:

SF2 = ab + acd + abcd + baed + dabec

Now repeat the process starting with the third term. Hence:

Step 3.1: Make the fourth term of SF2 (baed) disjoint with respect to the third (aiicd). The relative complement is the variable c so replace the fourth term by ECaed

Step 3.2 Make the fifth term of SF2 (adbec) disjoint with respect to the third (abcd). In this instance, the terms are already disjoint (on account of variables a and d, and according to the defined format), so there is nothing further to do.

The system failure expression at this stage is:

SF3 = ab + acd + ab cd + b caed + clabec

and since no further simplification appears possible, this is the final disjointed expression.

# Summary of Concerns:

With the understanding that the some or many users may or may not, know as much as they think they do about Boolean modeling or mathematics or expectation of the magnitude of the expected results, those would unassumingly use the reputable reliability commercial software tool to get a solution to their large problems such as electrical distribution grid or radar array systems, and end up with a numerical result that does not represent a true result of their problems.

Randomization and simulation only does not make up for Boolean reduction or truth tables or disjointing. It provides options/simulations on the different sates of the blocks in the flow diagram but does not make up for any measure of getting the correct results.

The test results of comparison of the same diagram on another, known to be a not disjoint tool, show that the PTC method is also not disjointed because the two provide results very close numerically.

The calculation tool in Flow RBD does not offer choice for calculations types at all, the only choice the user can make is with or without simulation. Disjointing, Boolean truth table reduction and such is not offered.

Providing disjointing or application of other Boolean techniques in this software tool is of paramount importance as the tool as it is now, provides results that cannot represent reliability and may lead to drastically incorrect results.